PHASE-Net

Analysis & Implementation Report

| Field | Detail |
| --- | --- |
| Project | KEIKO — vision-based physiological monitoring during human–robot collaboration |
| Model | PHASE-Net (main model, per supervisor's instruction) |
| Author | Elham Baradaran |
| Supervisor | Sakif |
| Date | 22–23 July 2026 |
| Repository | https://github.com/elhamBaradaran/rPPG |

# 1. Objective

Move from the hand-built classical baseline (POS, Wang et al. 2017) to a modern deep-learning rPPG model. The supervisor specified PHASE-Net — a lightweight, physics-grounded rPPG model — with RhythmFormer as an alternative. This session covered two tasks:

- Analyse the PHASE-Net paper.

- Plan and begin the implementation — get the model running locally.

# 2. Paper analysis

Reference: Zhao, B., Guo, D., Cao, J., Xu, Y., Zou, B., Tan, T., Sun, Y., Yu, Z. PHASE-Net: Physics-Grounded Harmonic Attention System for Efficient Remote Photoplethysmography Measurement. arXiv:2509.24850 — CVPR 2026 (Highlight). Code: https://github.com/Alex036225/PhaseNet

## 2.1 The core contribution: architecture derived from physics

The paper's central claim is that existing deep rPPG models are heuristic — architectures chosen by trial-and-error, which causes overfitting to dataset-specific noise and poor interpretability. PHASE-Net instead derives its architecture from first principles:

| Step | Physics | Result |
| --- | --- | --- |
| 1 | Beer–Lambert law + vessel compliance | Pixel intensity change is proportional to blood-volume change, which is proportional to pressure pulsation. The pulse z(t) is physically embedded in the video. |
| 2 | Navier–Stokes equations, linearised and 1-D averaged | A damped wave equation for pressure pulse propagation. |
| 3 | Single-point observation (fixed facial location) | Reduces to a second-order ODE — a forced damped harmonic oscillator: z''(t) + α·z'(t) + ω²·z(t) = u(t) |
| 4 | Discretised via semi-implicit Euler → LTI state-space model | Proposition 1: the solution is exactly a causal convolution of past inputs. |
| 5 | IIR → FIR approximation | Proposition 2: this is precisely what a Temporal Convolutional Network (TCN) computes. |

Conclusion: the TCN is not a design guess — it is the mathematically mandated architecture. The authors state this is the first work to build a theoretical bridge between the underlying physiological dynamics and a specific network architecture.

## 2.2 Architecture

Pipeline:  video → Vision Encoder (3 EST blocks) → Adaptive Spatial Filter → Gated TCN → pulse waveform

- ZAS (Zero-FLOPs Axial Swapper) — a parameter-free, reversible 2×2 block transpose applied to a subset of channels, mixing distant facial regions (forehead ↔ cheeks) at zero computational cost. Proven self-inverting and energy-preserving (1-Lipschitz), which stabilises training.

- ASF (Adaptive Spatial Filter) — learns a per-frame spatial softmax mask that highlights pulse-rich skin and suppresses noisy regions, then aggregates spatially and concatenates the first-order temporal derivative (vt = zt − zt−1) to encode local pulse dynamics.

- GTCN (Gated TCN) — causal dilated TCN with tanh × sigmoid gating. Ablation shows 3 layers is optimal (4 layers slightly degrades performance).

- Loss: Negative Pearson Correlation (waveform-shape similarity). Auxiliary loss weight λ = 0.1.

## 2.3 Reported results

Efficiency (Table 5 — 128×128, T = 128):

| Method | Params (M) | MACs (G) |
| --- | --- | --- |
| PHASE-Net | 0.29 | 28.3 |
| RhythmFormer | 4.21 | 28.8 |
| PhysFormer | 7.38 | 40.5 |
| PhysNet | 0.77 | 56.1 |
| DeepPhys | 7.50 | 96.0 |

Intra-dataset accuracy (MAE, bpm — lower is better):

| Method | UBFC-rPPG | PURE | BUAA | MMPD |
| --- | --- | --- | --- | --- |
| POS | 4.08 | 3.67 | — | 12.36 |
| PhysNet | 2.95 | 2.10 | 10.89 | 4.80 |
| RhythmFormer | 0.50 | 0.27 | 9.19 | 4.69 |
| PHASE-Net | 0.15 | 0.14 | 5.89 | 4.78 |

Cross-dataset generalisation (leave-one-out, MAE bpm) — the strongest result, and the most relevant to KEIKO, since a robot lab will not resemble the training data:

| Test on → | PHASE-Net | RhythmFormer | PhysFormer |
| --- | --- | --- | --- |
| PURE | 2.86 | 21.11 | 19.75 |
| BUAA | 2.56 | 6.04 | 22.09 |
| UBFC | 10.04 | 14.71 | 10.29 |
| MMPD | 10.33 | 16.14 | 13.90 |

PHASE-Net is roughly an order of magnitude better at transferring to PURE, supporting the claim that it learns pulse physics rather than dataset-specific appearance cues.

## 2.4 Relevance to KEIKO

- Robustness under motion and illumination change is the exact failure mode observed in our POS baseline (the estimate locked onto motion frequency and dropped to ~40 BPM under head movement).

- Lightweight and deployment-ready — important for eventual real-time use alongside the robot.

- Notably, PURE — one of the paper's benchmark datasets — is itself titled "Non-contact video-based pulse rate measurement on a mobile service robot", i.e. a human–robot interaction context directly analogous to KEIKO.

## 2.5 Decision: PHASE-Net over RhythmFormer

Both are built on the same rPPG-Toolbox framework, so implementation difficulty is comparable. PHASE-Net was retained because it is roughly 14× smaller in parameters, reports substantially better cross-domain generalisation, and was the supervisor's explicit choice. RhythmFormer remains a viable fallback within the same toolbox.

# 3. Implementation work completed

## 3.1 Environment

| Component | Value |
| --- | --- |
| OS | Windows 11 Pro |
| Python | 3.12.7 (existing project venv keiko-rppg\venv) |
| PyTorch | 2.6.0+cu124 — torch.cuda.is_available() == True |
| GPU | NVIDIA RTX 3050 Laptop — 4 GB VRAM, driver 592.00 (CUDA 13.1) |
| Added packages | torch, torchvision, thop, matplotlib |

Hardware constraint: the paper trained on a single NVIDIA H100. With 4 GB of VRAM, training from scratch is not feasible; the work therefore targets inference using the authors' pretrained weights, which is sufficient for evaluation and for the KEIKO use case.

## 3.2 Obtaining the code and weights

- Cloned the official repo to D:\00-TU-CLAUSTHAL\keiko-rppg\PhaseNet, deliberately outside the personal rPPG git repository to avoid nested-repository problems.

- Windows 260-character path limit blocked the clone: the repo contains a log directory with an extremely long name. Resolved with git config --global core.longpaths true followed by git restore --source=HEAD :/

- The repository ships genuine pretrained weights (not a Git-LFS placeholder): PhysLLM_PhaseNet_UBFC_UBFC_072_100_Epoch9.pth, 13,237,939 bytes, trained on UBFC-rPPG. Copied to the short path PhaseNet\weights\phasenet_ubfc_epoch9.pth to permanently avoid the long-path issue.

## 3.3 Milestone 1 — model and weights verified (01_load_phasenet.py)

Rather than invoking the full rPPG-Toolbox pipeline, the model class is imported directly. This matters because dataset/data_loader/BaseLoader.py unconditionally imports retinaface, which pulls in TensorFlow; importing only neural_methods.model.PhaseNet.PhaseNet avoids that heavy dependency entirely for inference.

Model constructed with the settings recovered from the config YAML and trainer source:

PhaseNet(feature_dim=128, latent_dim=32, hidden_dim=128, tcn_layers=4,

encoder_channels=[16, 32, 64, 128], encoder_expand_ratio=4,

temporal_module="gated_tcn")

Results:

| Check | Outcome |
| --- | --- |
| Weight loading | 0 missing, 0 unexpected keys — all 82 tensors matched exactly |
| Forward pass | Succeeded on GPU in 1.60 s |
| Peak GPU memory | 1.66 GB of 4.00 GB available — comfortable headroom |
| Input shape | (1, 3, 128, 128, 128) = (clip, RGB, 128 frames, 128×128 px) |
| Output shape | (1, 128) — one pulse value per frame |

The perfect key match confirms both that the architecture was reconstructed correctly and that the checkpoint is genuine and intact.

## 3.4 Milestone 2 — webcam inference script (02_webcam_phasenet.py)

The preprocessing was reverse-engineered from the repo source so that it matches training exactly:

- Frames read and converted BGR → RGB.

- Haar-cascade face detection on the first frame only (DO_DYNAMIC_DETECTION: False).

- Bounding box enlarged 1.5× about its centre (LARGE_BOX_COEF: 1.5).

- Crop, then resize to 128×128 with cv2.INTER_AREA.

- DATA_TYPE: Raw — pixel values kept in the 0–255 range. BaseLoader.__getitem__ performs np.float32(data) with no division by 255; normalising to 0–1 here would silently corrupt the input scale. (This was the single most important detail to get right.)

- Tensor laid out as NCDHW → (1, 3, 128, 128, 128).

- Chunked into 128-frame clips.

Heart rate is computed with the repo's own post-processing functions (_detrend, _calculate_fft_hr): detrend (λ = 100) → 1st-order Butterworth bandpass 0.75–2.5 Hz (45–150 BPM) → FFT peak → ×60.

Self-test results (no webcam required):

| Test | Result |
| --- | --- |
| BPM computation on a synthetic 75.0 BPM sine | 75.59 BPM — correct within FFT bin resolution |
| Full pipeline on 128 synthetic frames | Ran end-to-end on CUDA with no errors; shapes consistent |

# 4. Key technical findings

## 4.1 Parameter count does not match the paper's claim

The paper reports 0.29 M parameters. Measured directly from the released checkpoint:

total (whole checkpoint) : 3,300,274  (3.300 M)

training-only modules    : 2,487,984  (2.488 M)   <- Decoder1D + projection_encoder

inference path           :   812,290  (0.812 M)

paper claims             :   290,000  (0.290 M)

The Decoder1D and projection_encoder blocks are used only during training (they implement the reconstruction regularisation term) and can fairly be excluded from an inference-cost figure. Even so, the inference path holds 0.81 M parameters — approximately 2.8× the published 0.29 M.

This does not invalidate the paper's accuracy results, and PHASE-Net remains markedly smaller than RhythmFormer (4.21 M) and PhysFormer (7.38 M). It is, however, a reproducibility discrepancy worth raising, and it may indicate that the reported figure corresponds to a different or reduced configuration than the released checkpoint.

## 4.2 Released checkpoint uses 4 TCN layers, not the ablation-optimal 3

The paper's ablation concludes that 3 TCN layers is optimal on UBFC-rPPG (MAE 0.15 vs 0.25 at 4 layers) and states "we set the default depth to 3". The released checkpoint contains temporal_model.network.{0,1,2,3} — i.e. 4 layers. A minor inconsistency between the published ablation and the released artefact.

## 4.3 Practical / reproducibility notes

- The retinaface dependency (and thus TensorFlow) is required only for the RF face-detection backend and the data-loading pipeline; the configs use the lightweight HC (Haar cascade) backend, so it can be avoided for inference.

- Windows long-path handling is a genuine blocker for this repository and must be enabled before cloning.

# 5. Current status

| Item | Status |
| --- | --- |
| Paper analysed | Complete |
| PyTorch + CUDA environment | Working (GPU confirmed) |
| Repository cloned | Complete |
| Pretrained weights obtained and validated | Complete (perfect key match) |
| Model runs on local GPU | Complete (1.66 GB peak, well within 4 GB) |
| Preprocessing replicated from source | Complete |
| Webcam inference script + self-test | Written and self-tested |
| Live webcam measurement on a real face | Pending — not yet executed |
| Validation against UBFC-rPPG ground truth | Not started (dataset not yet obtained) |
| Full rPPG-Toolbox only_test reproduction | Deferred (requires dataset + TensorFlow) |
| Training / fine-tuning | Not feasible on 4 GB VRAM |

No rPPG dataset (UBFC-rPPG, PURE, MMPD) is currently present on the local machine. All results above are therefore either from the released checkpoint or from synthetic self-tests; no accuracy figure has yet been reproduced against ground truth.

# 6. Next steps

- Run the webcam script on a real face and compare against the Samsung smartwatch reference — the same informal validation used for the POS baseline (which gave ~78 BPM vs the watch's 74 BPM).

- Obtain UBFC-rPPG to enable quantitative validation against the CMS50E ground truth, and attempt to reproduce the paper's reported 0.15 bpm MAE.

- Compare PHASE-Net against the POS baseline under identical conditions — particularly under head motion, where POS is known to fail — to quantify the improvement for the KEIKO scenario.

- Adapt to the KEIKO setting: longer recordings, subject seated at the collaborative task, and eventually a sliding-window real-time mode.

- Longer term: heart rate variability (HRV), which requires accurate beat-to-beat peak timing rather than only an averaged BPM.

# 7. Files produced

| File | Purpose |
| --- | --- |
| Models/PHASE-Net/01_load_phasenet.py | Loads the model + pretrained weights, verifies key match, reports parameter counts, runs a timed GPU forward pass |
| Models/PHASE-Net/02_webcam_phasenet.py | End-to-end webcam → face crop → PHASE-Net → BPM, with --selftest mode |
| Models/PHASE-Net/README.md | GitHub-facing README: same content plus setup and run instructions |
| PhaseNet/weights/phasenet_ubfc_epoch9.pth | Pretrained UBFC checkpoint, copied to a short path |

# 8. References

[1]  Zhao, B. et al. PHASE-Net: Physics-Grounded Harmonic Attention System for Efficient Remote Photoplethysmography Measurement. CVPR 2026 (Highlight). arXiv:2509.24850.

[2]  Liu, X. et al. rPPG-Toolbox: Deep Remote PPG Toolbox. arXiv:2210.00716, 2022.

[3]  Wang, W., den Brinker, A. C., Stuijk, S., de Haan, G. Algorithmic Principles of Remote PPG. IEEE TBME 64(7), 2017. — the POS baseline.

[4]  Bobbia, S. et al. Unsupervised skin tissue segmentation for remote photoplethysmography. Pattern Recognition Letters 124, 2017. — UBFC-rPPG dataset.

[5]  Zou, B. et al. RhythmFormer: Extracting patterned rPPG signals based on periodic sparse attention. Pattern Recognition 164, 2025. — considered alternative.